WIRESHARK ANALYSIS CHECKLIST

wireshark(1) Wireshark Manual Page – Interactively dump and analyze network traffic

- What Conversations are taking place?

- Statistics > Conversations

- What devices are identified on this network?

- Statistics > Endpoints

- What ports are the devices communicating on?

- Statistics > IPv4

  - Destinations and Ports

- Which protocols are being used on the network?

- Statistics > Protocol Hierarchy

- Are there any external (public) IP addresses?

- Ip.src != 192.168.x.x/24 or ip.dst != 192.168.x.x/24

- What are they (reverse DNS lookup)?

  - Network Tools: DNS,IP,Email (mxtoolbox.com)

- Are they an NIH IP?

- What are the external IP’s talking to?

- Ip.addr == ipa.ddr.ess

- Any OT specific traffic

  - DNP 3

  - Modbus

  - IEC60870-5

  - SNMP

  - Hart

  - CIP

  - BACnet

  - ICCP / Tase

  - Profinet (all types)

    - dnp3 or modbus or iec60870_5_103 or snmp or hart_ip or cip or bacnet or tcp.port == 102 or dcom or pn_dcp or pn_io or pn_mrp or pn_mrrt or pn_ptcp or pn_rt

- Any TCP traffic between devices?

  - Tcp

    - What traffic is observed?

- Observe the entire communication for data

  - Right click packet > Follow > TCP Stream

- Any UDP Traffic between devices?

  - Udp

    - What traffic is observed?

- Observe the entire communication for data

  - Right click packet > Follow > UDP Stream

TShark

Tshark | tshark.dev

tshark(1) (wireshark.org) MANUAL PAGE  tshark- Dump and Analyze Network Traffic

- tshark -n -w FILENAME.pcap -q -z endpoints, ip (Statistic > Endpoints)

- tshark -n -w FILENAME.pcap -q -z endpoints, tcp (TCP Ports being used)

- tshark -n -w FILENAME.pcap -q -z conv, ip (IP to IP Conversations)

- tshark -n -w FILENAME.pcap -q -z conv, eth (MAC to MAC Conversations)

- tshark -n -w FILENAME.pcap -z io, phs (Protocol Hierarchy)?????